- Binary HD Matrix

- Crestron Module Help File

Description

These modules will support a RS-232 or TCP-IP connection to a Binary HD Matrix switcher.

Supported Binary Products

These modules were tested against a B100 8X8 switcher running firmware version 1.00.12.

Supported Processors

These modules will run on any Crestron 2-series or 3-series processor.

Supported Applications

These modules have been tested in SIMPL Windows as well as SystemBuilder.

Release Notes

- [V1] - 8-22-2012 - Release candidate

- Serial Pinouts

- The connection requires a straight through cable constructed according to this table.  Do not connect any other pins.  Do not use a factory made cable unless you know that only pins 2-3-5 are populated.

| - Cable End - Crestron |  | - Cable End - Switcher |
| --- | --- | --- |
| - Female DB-9 |  | - Female DB-9 |
| - 2 | - > | - 2 |
| - 3 | - > | - 3 |
| - 5 | - > | - 5 |

- Comport settings

Baud rate: 9600

Data bits: 8

Stop bits: 1

Parity: None

Hardware handshaking: None

Software handshaking: None

System Builder Support

Once the module is added to SystemBuilder, you will need to make all the appropriate connections to the system logic and touch panel template that you are using.

In order for the module to function you first have to drop these modules into your default User Module path.

Binary_HDMatrix(Ethernet)_v1.umc

Binary_HDMatrix(Serial)_v1.umc

This path can be found under EDIT>PREFERENCES>USER DATABASE PATHS. Once you have placed the modules in the appropriate folder, be sure to click rebuild.

Next open up your project and select the Equipment view.  In the lower right hand corner open the User Database and drill down the By Device Type until you see SnapAV. Expand the category until you see the B-100-HDMatrix(Ethernet) or B-100-HDMAtrix(Serial) and select add to system.

Once you have added the object to your program, you have to setup the parameters for the module.  Right click on the object and select Properties.  Then select I/O Assignment from the left hand pane.  Here you should verify that the Serial or IP settings are correctly set on the I/O tab. Also, for the Ethernet version, make sure to fill out the parameter values on the properties tab.

Next select Audio from the left hand pane and verify that this is NOT defined as a distributed audio source.

Signal and Parameter Descriptions

DIGITAL INPUTS

[connect_fb>>tcp-ip_client]	Connect to the output of the TCP-IP client

[tcp-ip_connect]	Pulse to make a TCP-IP connection to the switcher

[tcp-ip_disconnect]	Pulse to disconnect the TCP-IP connection from the switcher

[power_on]	Pulse to power on the switcher

[power_off]	Pulse to power off the switcher

[output1-x_selection_+]	Pulse to increment the input feeding output #1

[output1-x_selection_-]	Pulse to decrement the input feeding output #1

[output1-x_on]	Pulse to turn the #1 HDMI output on

[output1-x_off]	Pulse to turn the #1 HDMI output off

[input_n1-16]	Pulse to select this input as the “x” of your “xy” selection

[output_n1-16]	Pulse to select this input as the “y” of your “xy” selection

[xy_take]	Pulse to update the switcher with the “x” and “y” values from the above two digitals

[status_poll]	Pulse to have the switcher update the module with the current settings for each output

[firmware_poll]	Pulse to have the switcher update its firmware version as well as series and model values

ANALOG INPUTS

Matrix_tcpip_status	(Ethernet version only) Route from the TCP-IP client symbol for the matrix. Tracks connection status between the Crestron processor and the matrix

[output1-8]	Feed an analog value from a symbol such as an INIT to force the switcher output to a new input

SERIAL INPUTS

switcher_rx$	Connect to the rx$ line of the com port or tcp-ip client

DIGITAL OUTPUTS

[client_connect>>tcp-ip_client_connect]	(Ethernet version only)Tie to the connect line of the TCP-IP client

[matrix_tcp-ip_logged_in]	(Ethernet version only) Held high when tcp-ip client is connected and the module has successfully logged in to the switcher

[matrix_tcp-ip_not_logged_in]	(Ethernet version only) Held high when the module did not successfully log in to the switcher

[power_on_fb]	High if the switcher is powered on

[power_off_fb]	High if the switcher is powered off

[output1-16_on_fb]	High if the output is enabled

[output1-16_off_fb]	High if the output is disabled

[input_n1-16_fb]	High when this input is selected as the “x” of your “xy” selection

[output_n1-16_fb]	High when this input is selected as the “y” of your “xy” selection

[product_series_b100]	High to indicate this product series

[product_series_b300]	High to indicate this product series

[product_series_b500]	High to indicate this product series

[product_model_4x4]	High to indicate a 4x4 model switcher

[product_model_8x8]	High to indicate a 8x8 model switcher

[product_model_8x16]	High to indicate a 8x8 model switcher

[product_model_16x16]	High to indicate a 8x8 model switcher

[command_error]	Pulses high for 1s when the switcher does not acknowledge the transmitted string

ANALOG OUTPUTS

[output1-8_feedback]	Current analog value of the input assigned to this output

SERIAL OUTPUTS

switcher_tx$	Connect to the tx$ line of the com port

PARAMETERS (Ethernet version only)

username	Enter the TCP-IP login credentials in ASCII for the switcher

password	Enter the password in ASCII characters that for the above TPC_IP login

Tcp_port	This parameter has only one valid value and is used for SystemBuilder support