SnapAV Visualint ELAN Driver

V1.0 rev20171211

Overview

This driver integrates a SnapAV Visualint VIM Camera into your ELAN control system. The driver was created and tested on a gSC2 series controller using g!Viewer. To use this driver, add the driver to the system, and follow the setup.

Specification

This package includes the following:

- Compiled ELAN Driver (SnapAV_Visualint_VIM_Control.EDRVC)

  - Turn On/Off VCA rules

  - Turn On/Off video streams

  - Event mappable triggers from Visualint Events

IP Connection

The driver communicates to and from the device through an IP connection and must be connected for the driver to function.

EVENTS (OPTIONAL):

1. Enter the camera's configuration.

2. Under "Alarm Management", select "Visualint Event Output".

3. Create a "New" event.

4. Enter any "Name" and "Description" you want.

5. Enter the address of the ELAN Controller with this driver in "Address".

6. Enter the port listed in the "Event Port" property in "Port".

(NOTE: This port is randomly generated when the driver is added to the project.

It is changeable, and must be unique for each camera driver. Also requires an update driver click.)

7. Leave "Account" and "Password" blank.

8. In the "Message" field, enter a number from 1 to 10, this will trigger the

corresponding “Event Trigger X“ on the ELAN Controller. Entering anything other than

1-10 will trigger the "Event Trigger Custom" event on the ELAN Controller driver.

(NOTE: There is only one "CUSTOM" event available, so multiple custom "Visualint

Events" will only trigger the same Control4 Event.)

9. Press "Test", and if the result returns "Success", you can "Save".

10. Repeat steps 3 through 9 for every event you wish to create.

Setup

1.	After setting up the Visualint VIM Camera, add the driver to the controller as a "new Lighting Interface" in g!Tools Configurator under the "Lighting"  menu.

2.	Enter the IP Address of the Visualint Vim Camera into the "IP Address" field and the Port in the "Port" field in the driver's configuration. Default Port is 80.  Enter the username and password of the Visualint VIM Camera. Default username is admin. Default password is 1111. Once connected, the driver will auto configure and add a “on/off Lighting device” for vca rules, streams, and event triggers.

3.	Lastly, under “Custom Pages”, add a new page for the Visualint source and create “Light Toggle Control” buttons. These buttons can then be tied to the newly created On/Off light device.